JAX RomeroNNV Model Implmentation Methods

We recreated a hybrid neural ordinary differential equation model based on the RomeroNNV formulation described by Wang. The model represents the plasma state as

xt=lit,Ipt,Vt

Where li is the internal inductance, Ip is the plasma current in mega-amperes, and V=Vc-Vb is the internal voltage-like state. The external control input was the inductive drive signal Vind, corresponding to Vind=Vb-Vr.

The model preserves the analytic Romero physics for the internal inductance and plasma-current dynamics, while replacing the analytic voltage-state equation with a neural network approximation. Specifically, the derivatives of li and Ip were computed as

li=αli∙-2Vind-2VC∙Ip

Ip=αIp∙2Vind+VC∙li

where C = 3.6559, αli & αIp are trainable constants introduced to compensate for normalization constants and unit scaling present in the original implementation but not explicitly available in the manuscript. These constants were implemented as trainable JAX arrays and optimized jointly with the neural network parameters.

The voltage-state derivative was modeled using a fully connected neural network,

V=fθli,Ip,V,Vind

Where fθ∙  was an Neural Network with four inputs, one output, hidden width 32, depth 2, and softplus activation. Thus, the neural network only learned the unknown voltage dynamics, while the known physical structure for li and Ip was retained.

The hybrid derivative model returned

xt=lit,Ipt,Vt

which was integrated forward in time using a 4th-order Runge-Kutta scheme. The control signal Vind was interpolated between measured samples using cubic Hermite interpolation with backward-difference slopes, preserving causality in the control trajectory.

Experimental plasma shots were split at the shot level into training, validation, and test sets using a 70/15/15 split. Each shot was subsequently divided into non-overlapping shooting windows of length 10 time samples using stride 10. Each window was initialized from the measured plasma state at the beginning of the segment, and trajectory loss was computed independently over each shooting interval. No continuity penalty was enforced between adjacent windows, corresponding to a zero-continuity-weight approximation of the multiple-shooting strategy described by Wang.

The plasma shots consisted of irregularly sampled time-series measurements of for li, Ip, V, & Vind. Rather than resampling all trajectories onto a uniform temporal grid during training, the original experimental sampling times were retained. Multiple-shooting segments were therefore defined directly using contiguous intervals of the measured time array. The neural ODE solver propagated the system using the local measured timestep sequence,

Δtk = tk+1−tk,

allowing the model to preserve the irregular temporal structure of the experimental discharges.

To evaluate the control signal between measured samples during Runge–Kutta integration, the inductive drive signal Vind was interpolated using cubic Hermite interpolation with backward-difference slope estimates. This interpolation strategy preserved causality while producing a continuously differentiable approximation of the control trajectory during numerical integration.

For diagnostic analysis, comparison plotting, and residual analysis, trajectories were additionally interpolated onto regularized temporal grids using linear interpolation. These regularized grids were used only for post-training analysis and were not used during model optimization.

The loss function followed the Wang-style objective by comparing only the predicted li and Ip trajectories to measured data. For each segment, relative error was computed for li and Ip transformed using a Huber loss with delta = 0.1, and integrated over time using the trapezoidal rule. The voltage state V was not directly included in the primary training loss.

Training was performed using AdamW in Optax with an exponential learning-rate schedule initialized at 5 x 10-3, decay rate 0.995 per epoch, and lower bound 1 x 10-4. Gradient clipping and NaN filtering were used for numerical stability. The full set of training shooting segments was included in each batch. The best model was selected using validation loss.

For comparison, we also implemented the analytic Romero model in which the neural network voltage equation was replaced by the first-order analytic expression

V=-Vτ-κτVind

with κ = 0.98 and τ = 1.25. To isolate the effect of the voltage model, the analytic Romero comparison model used the same learned αli & αIp scaling constants as the trained RomeroNNV model. Thus, differences between RomeroNNV and analytic Romero were primarily attributable to the learned versus analytic representation of V.